Design and Performance Evaluation of a Secure Smart Hospital Network for Telemedicine and Critical Care

A CAPSTONE PROJECT REPORT

A Capstone Project Report submitted in partial fulfilment of the requirements for the Course of

CSA0703 — Computer Networks

to the award of the degree of

BACHELOR OF ENGINEERING / BACHELOR OF TECHNOLOGY

IN

ARTIFICIAL INTELLIGENCE AND MACHINE LEARNING

Submitted by

Harsheni J (1921260080)

Under the Supervision of

Dr.JAISHARMA

SIMATS ENGINEERING

Saveetha Institute of Medical and Technical Sciences

Chennai-602105

SEPTEMBER 2026

SIMATS ENGINEERING

Saveetha Institute of Medical and Technical Sciences

Chennai-602105

BONAFIDE CERTIFICATE

This is to certify that the Capstone Project entitled “Design and Performance Evaluation of a Secure Smart Hospital Network for Telemedicine and Critical Care” has been carried out by Harsheni .J (1921260080) under the supervision of Guide Name and is submitted in partial fulfilment of the requirements for the current semester of the B.Tech.Artificial Intelligence and Machine Learning program at Saveetha Institute of Medical and Technical Sciences, Chennai.

| COURSE COORDINATOR Title / Name, Designation Department of CSE SIMATS Engineering, Chennai – 602105. | COURSE FACULTY Dr.JAISHARMA Designation Department of CSE SIMATS Engineering, Chennai – 602105. |
| --- | --- |

SIMATS ENGINEERING

Saveetha Institute of Medical and Technical Sciences

Chennai-602105

DECLARATION

We, Harsheni .J  of the Department of  IT , SIMATS Engineering, Saveetha Institute of Medical and Technical Sciences, Chennai, hereby declare that the Capstone Project Work entitled “Design and Performance Evaluation of a Secure Smart Hospital Network for Telemedicine and Critical Care” is the result of our own bonafide efforts. To the best of our knowledge, the work presented herein is original, accurate, and has been carried out in accordance with the principles of engineering ethics and academic integrity. All sources, references, data, and other materials used in the project have been appropriately acknowledged. We further declare that the data, results, analysis, and findings presented in this report have not been fabricated, falsified, or manipulated. Any external tools, software, datasets, computational resources, or AI-assisted tools used during the project have been appropriately disclosed. We confirm that all applicable ethical, academic, institutional, and professional requirements have been duly followed throughout the planning, execution, analysis, and documentation of the project.

Place: Chennai

Date: _______________

Signature of the Students with Names

Individual Contribution Statement

(Mandatory for all team projects)

| Student | Specific Responsibilities | Design & Development Contribution | Testing & Analysis Contribution | Approx. Contribution (%) |
| --- | --- | --- | --- | --- |
| HARSHENI.J | Overall network design, Packet Tracer topology, addressing plan, VLANs, wireless APs | Designed hybrid topology, addressing plan, VLAN scheme, Wi-Fi 6 AP layout | Verified end-to-end connectivity and VLAN isolation in Packet Tracer | ~33% |
| JACK YANNIS.F | TCP/UDP/QUIC discussion, congestion & QoS analysis, Wireshark captures, performance metrics | Designed QoS/priority-queuing policy and transport-layer test scenarios | Captured and analysed Wireshark traces; computed throughput/delay/loss metrics | ~33% |
| JAICHITRA.G | DNS, HTTP/HTTPS+TLS, SMTP/FTP, SNMP; GitHub repo setup, report compilation, reflection | Designed application-layer services and security configuration | Verified DNS/TLS/SMTP/SNMP behaviour via Wireshark; collected evidence from all members | ~34% |
| Total |  |  |  | 100% |

ABSTRACT

Hospitals increasingly depend on real-time telemedicine, continuous IoT-based patient monitoring, and secure transfer of large medical imaging files, yet legacy network designs often cannot meet the reliability, latency, and security demands these services impose. This project addresses the modernisation of a three-building multi-specialty hospital network (Emergency & ICU Block, Diagnostic & Imaging Block, and Administrative & OPD Block) that currently suffers from high wireless packet loss near MRI rooms, slow routing convergence during equipment failures, TCP retransmission storms at peak telemedicine load, and insecure or unreliable application services.

A hierarchical, hybrid star/partial-mesh topology was designed and implemented in Cisco Packet Tracer, using Wi-Fi 6 (802.11ax) wireless access, VLAN-segmented departments, and a fibre backbone sized for 500 Mbps inter-building imaging transfer. Transport-layer performance was evaluated by comparing TCP, UDP, and QUIC behaviour under simulated hospital traffic patterns, with priority queuing applied to critical-care traffic and validated through Wireshark packet captures. Application-layer services — DNS, HTTPS with TLS 1.3, secure file transfer for DICOM images, SMTP alerting, and SNMPv3 monitoring — were configured and verified against the stated security and privacy requirements.

Results captured from Packet Tracer Simulation Mode and Wireshark demonstrate improved DNS resolution latency, reduced packet loss in previously problematic wireless zones, faster routing convergence after simulated link failure, and measurable prioritisation of emergency traffic over routine transport. The work confirms that a layered, evidence-driven redesign — combining data-link error control, hierarchical addressing, transport-layer QoS, and secured application services — can materially improve the reliability, security, and patient-safety posture of a hospital network while remaining implementable with standard simulation tools.

Keywords:

Hospital network design, telemedicine, Wi-Fi 6, QoS and congestion control, network security, DNS/TLS, Packet Tracer, Wireshark.

TABLE OF CONTENTS

| Sl. No | Title | Page No |
| --- | --- | --- |
| i | Certificate from the Supervisor | ii |
| ii | Declaration by the Candidates | iii |
| iii | Individual Contribution Statement | iv |
| iv | Abstract | v |
| 1 | Chapter 1: Introduction | 1 |
| 2 | Chapter 2: Literature Review | 5 |
| 3 | Chapter 3: Engineering Design & Trade-offs, Sustainability, Ethics, Safety, Risk & Security | 8 |
| 4 | Chapter 4: Implementation, Testing & Results, Project Management | 16 |
| 5 | Chapter 5: Conclusion & Reflection | 22 |
|  | References | 24 |
|  | Appendices | 25 |

# CHAPTER 1: INTRODUCTION AND ENGINEERING PROBLEM

## 1.1 Background

Modern hospitals rely on networked systems for far more than administrative record-keeping: real-time telemedicine consultations, continuous monitoring of IoT-enabled medical devices such as patient monitors and infusion pumps, and secure transfer of large diagnostic imaging files are now core clinical functions. A multi-specialty hospital spanning three buildings — Emergency & ICU Block, Diagnostic & Imaging Block, and Administrative & OPD Block — depends on its network performing reliably enough that a delay or failure does not become a patient-safety incident.

## 1.2 Need for the Project

- Why it needs addressing: the existing network shows high packet loss in wireless zones near MRI rooms, delayed routing convergence after equipment failure, TCP retransmission storms during peak telemedicine hours, and insecure or slow application services (DNS delays, unencrypted file transfers, email alert failures).

- Who is affected: clinical staff relying on real-time monitoring and telemedicine, patients whose care depends on timely alerts and imaging access, and administrative staff dependent on reliable record and communication systems.

- Existing limitations: flat or poorly segmented addressing, routing that does not converge fast enough for a clinical setting, no differentiated treatment of critical-care traffic, and application services configured without adequate security or redundancy.

- Engineering significance: the problem requires coordinated design across the data-link, network, transport, and application layers rather than a single fix, since a failure at any layer (e.g., wireless interference, slow routing, congestion, or an insecure protocol) can independently degrade patient care.

## 1.3 Problem Statement

Design, implement (via simulation), and evaluate an end-to-end hospital network across three buildings that achieves high reliability and low latency for critical-care traffic, strong security for patient data, and scalability to at least 2,000 concurrent wireless clients and 300 IoT medical sensors — while resolving current wireless packet loss, routing convergence delays, transport-layer congestion, and insecure/unreliable application services, under realistic constraints of cost, cabling, and hospital operational continuity.

## 1.4 Project Aim

To design and simulate a secure, high-performance, and scalable network architecture for a multi-specialty hospital that reliably supports telemedicine, IoT patient monitoring, medical imaging transfer, and administrative services.

## 1.5 Project Objectives

- 1. To design a hierarchical, hybrid network topology connecting the three hospital buildings with redundant inter-building links.

- 2. To develop an addressing and VLAN segmentation plan per department that supports the stated device and client scale.

- 3. To configure and compare routing protocols for fast convergence after link failure.

- 4. To model and evaluate TCP, UDP, and QUIC behaviour under hospital traffic patterns, including a priority-queuing scheme for critical-care traffic.

- 5. To implement secured application-layer services (DNS, HTTPS/TLS 1.3, secure file transfer, SMTP, SNMPv3) that meet the stated privacy and reliability requirements.

- 6. To test and validate the resulting design using Packet Tracer Simulation Mode and Wireshark, and to evaluate results against the stated performance targets.

## 1.6 Scope

- Included: topology and addressing design, VLAN segmentation, Wi-Fi 6 wireless access design, routing configuration, transport-layer QoS analysis, and application-layer service design and security for DNS, HTTP/HTTPS, file transfer, SMTP, and SNMP.

- Excluded: physical procurement and installation, integration with a real hospital's live clinical systems, and legal/regulatory certification (the project discusses HIPAA-like considerations conceptually, not as a compliance audit).

- Assumptions: device counts and traffic patterns are based on the figures given in the assignment problem statement; simulated rather than production hardware is used throughout.

- Boundary conditions: the design targets the stated performance thresholds (packet loss < 0.5% in critical wireless zones, DNS resolution < 30 ms local, routing convergence < 3 s, TCP goodput recovery within 2 RTT) as design goals validated in simulation, not as guarantees for a production deployment.

## 1.7 Expected Engineering Outcome

A validated network design (topology diagrams, addressing/VLAN plan, device configurations) implemented in Cisco Packet Tracer, with supporting Wireshark evidence, delivered as a GitHub repository and this consolidated technical report — functioning as a simulated prototype network rather than a physical system.

# CHAPTER 2: LITERATURE REVIEW AND EXISTING SOLUTIONS

## 2.1 Review Method

Relevant material was identified from IEEE standards documentation (802.11ax/Wi-Fi 6), RFC documents for DNS, TLS 1.3, SMTP, and SNMPv3, Cisco Packet Tracer and Wireshark official documentation, and published discussion of hospital/healthcare network design practices and telemedicine infrastructure requirements.

## 2.2 Review of Existing Work

Traditional hospital networks are frequently built as flat or lightly segmented Ethernet/Wi-Fi networks without differentiated handling of clinical versus administrative traffic. Standard enterprise Wi-Fi (802.11n/ac) deployments struggle in RF-dense clinical areas and near MRI shielding, motivating the move to Wi-Fi 6/802.11ax, which introduces OFDMA and Target Wake Time (TWT) specifically aimed at dense-client and power-constrained IoT scenarios. On the transport side, published comparisons of TCP, UDP, and the newer QUIC protocol show QUIC's reduced connection-establishment latency and improved behaviour on lossy links, relevant to real-time telemedicine video. On the security side, healthcare-specific guidance consistently recommends TLS 1.3 for patient-facing web services and authenticated/encrypted SNMPv3 over legacy SNMP versions, given the sensitivity of patient data.

## 2.3 Comparative Analysis

| Existing Approach | Advantages | Limitations | Relevance to Proposed Work |
| --- | --- | --- | --- |
| Flat/lightly segmented LAN with legacy Wi-Fi (802.11n/ac) | Simple to configure; low initial cost | Poor performance in dense/RF-hostile clinical zones; no traffic isolation | Baseline against which the proposed VLAN + Wi-Fi 6 design is compared |
| Best-effort routing without QoS | Straightforward routing configuration | No prioritisation of emergency traffic; congestion affects all traffic equally | Motivates the priority-queuing design in Chapter 3 |
| Plaintext or legacy-secured application services (FTP, SNMPv1/v2c) | Lower configuration overhead | Credentials/data exposed in transit; unsuitable for patient data | Directly addressed by the TLS 1.3 / SFTP / SNMPv3 design in Chapter 3 |

## 2.4 Research / Engineering Gap

Existing hospital network guidance tends to treat wireless upgrade, routing redundancy, transport QoS, and application security as separate initiatives. There is limited published, simulation-validated demonstration of all four layers being designed together against a single, realistic hospital scenario with explicit numeric performance targets.

## 2.5 Proposed Contribution

This project contributes an integrated, simulation-validated design spanning the data-link through application layers for a specific, constraint-driven hospital scenario, with each design decision traced to a stated requirement (e.g., MRI-zone packet loss, DNS latency, routing convergence) and backed by Packet Tracer and Wireshark evidence rather than description alone.

# CHAPTER 3: ENGINEERING DESIGN & TRADE-OFFS, SUSTAINABILITY, ETHICS, SAFETY, RISK & SECURITY

## 3.1 Requirements

### Stakeholder / User Requirements

| Stakeholder | Requirement |
| --- | --- |
| Clinical staff (ICU/Emergency) | Low-latency, highly reliable connectivity for patient monitors, alerts, and emergency telemedicine |
| Radiology/Imaging staff | High-bandwidth, secure transfer of large DICOM imaging files between buildings |
| Administrative/OPD staff | Reliable email/alerting and standard application access |
| Patients (telemedicine) | Accessible, secure telemedicine portal with acceptable video quality |
| IT/network administrators | Manageable, monitorable, and secure infrastructure with fast fault recovery |

### Functional & Non-Functional Requirements

| Requirement Type | Measurable Target |
| --- | --- |
| Functional — capacity | Support ≥ 2,000 concurrent wireless clients and 300 IoT medical sensors |
| Functional — imaging transfer | Secure inter-building file transfer up to 500 Mbps |
| Non-functional — wireless reliability | Average packet loss < 0.5% in critical wireless zones |
| Non-functional — name resolution | DNS resolution < 30 ms (local) |
| Non-functional — congestion recovery | TCP goodput recovery within 2 RTT after congestion |
| Non-functional — fault tolerance | Routing convergence < 3 s for a single link failure |

## 3.2 Constraints & Applicable Standards

| Standard / Code | Requirement | How Applied in This Project |
| --- | --- | --- |
| IEEE 802.11ax (Wi-Fi 6) | OFDMA, MU-MIMO, Target Wake Time for dense/IoT wireless deployments | Selected as the WLAN standard for all wireless APs; TWT applied to wearable sensors |
| RFC 8446 (TLS 1.3) | Secure, low-latency transport encryption | Used for the telemedicine portal (HTTPS) and imaging transfer service |
| RFC 3411–3418 (SNMPv3) | Authenticated and encrypted network management | Used instead of SNMPv1/v2c for device and infrastructure monitoring |
| HIPAA-like privacy principles | Confidentiality and integrity of patient-related data in transit and at rest | Informs the choice of encrypted transfer, TLS, and access segmentation via VLANs |

## 3.3 Design Specifications

| Parameter | Target/Requirement | Unit | Priority |
| --- | --- | --- | --- |
| Wireless packet loss (critical zones) | < 0.5 | % | High |
| DNS resolution (local) | < 30 | ms | High |
| Routing convergence | < 3 | s | High |
| TCP goodput recovery | ≤ 2 | RTT | Medium |
| Inter-building imaging bandwidth | 500 | Mbps | High |
| Concurrent wireless clients | 2,000 | clients | Medium |

## 3.4 Alternative Solutions & Evaluation

Alternative 1: Pure star topology with a single central core connecting all three buildings over one fibre link each, using best-effort routing and legacy Wi-Fi.

Alternative 2: Hybrid star/partial-mesh topology with redundant fibre between buildings, VLAN-segmented departments, Wi-Fi 6 wireless access, and priority-queued transport for critical-care traffic (the selected design).

| Criterion | Weight | Alternative 1 (Star, best-effort) | Alternative 2 (Hybrid, QoS-aware — selected) |
| --- | --- | --- | --- |
| Performance | 30% | Moderate — no traffic prioritisation | High — priority queuing for critical traffic |
| Reliability | 25% | Low — single point of failure per building link | High — redundant inter-building paths |
| Security | 20% | Low — flat network, limited isolation | High — VLAN-segmented, encrypted services |
| Cost | 15% | Lower initial cost | Moderate — extra fibre links and Wi-Fi 6 hardware |
| Scalability | 10% | Limited — flat addressing does not scale cleanly | High — hierarchical addressing per department |

## 3.5 Selected Design & Detailed Design

Alternative 2 was selected: it scores higher on performance, reliability, security, and scalability, which the evaluation matrix in 3.4 weights most heavily for a hospital environment, at an acceptable increase in cost — a justified trade-off given the patient-safety stakes described in the problem statement.

### Topology and Addressing (Student A)

A hybrid star/partial-mesh backbone connects core routers in each building over redundant single-mode fibre; within each building, a star topology runs from distribution switches to access switches and Wi-Fi 6 APs. Hierarchical IPv4 addressing is applied per building and department:

| Building | Department / VLAN | Example Subnet | Approx. Hosts |
| --- | --- | --- | --- |
| Emergency & ICU Block | ICU Monitors / IoT (VLAN 10) | 10.10.0.0/22 | ~500 |
| Emergency & ICU Block | Staff Wi-Fi (VLAN 40) | 10.10.4.0/23 | ~400 |
| Diagnostic & Imaging Block | Imaging Servers (VLAN 30) | 10.20.0.0/23 | ~300 |
| Administrative & OPD Block | Admin/OPD (VLAN 50) | 10.30.0.0/23 | ~350 |
| Administrative & OPD Block | Guest/Patient Wi-Fi (VLAN 60) | 10.30.2.0/24 | ~200 |

Wi-Fi 6 APs are placed just outside MRI-shielded rooms rather than relying on wireless penetration through RF shielding, directly addressing the stated packet-loss issue in that zone; OFDMA supports dense simultaneous client service in nursing stations and waiting areas, and Target Wake Time is enabled for battery-powered wearable sensors.

### Transport-Layer Design & QoS (Student C)

Hospital traffic is grouped into three classes for transport-layer treatment: (1) critical-care telemetry and emergency telemedicine video, (2) bulk imaging transfer, and (3) routine administrative traffic.

- TCP is used for reliable, non-real-time traffic such as medical records and imaging transfer, where retransmission and ordering guarantees matter more than instantaneous delivery.

- UDP/QUIC is used for real-time telemedicine video and telemetry, where QUIC's faster handshake and improved loss recovery over UDP reduce the impact of the wireless packet loss identified near MRI zones, compared with plain UDP or TCP for the same traffic.

- Priority queuing (QoS) is applied at core/distribution switches so that VLAN 10/20 (ICU monitors, emergency telemedicine) traffic is scheduled ahead of VLAN 30 (imaging) and VLAN 40/50 (staff/admin) traffic during congestion, directly targeting the stated TCP retransmission storms during peak telemedicine hours.

- A simple M/M/1 priority-queue model is used to estimate expected queuing delay for critical-care traffic versus best-effort traffic under peak load, to justify the queuing configuration before implementation.

### Application-Layer Design & Security (Student D)

Application services are designed with security and the stated performance targets as first-class requirements:

- DNS: two-tier hierarchy — one authoritative server in the Administrative Block, one caching server per building — so most queries resolve locally, addressing the < 30 ms local resolution target and the stated DNS lookup delay problem.

- HTTPS/TLS 1.3: the telemedicine portal is served exclusively over HTTPS with TLS 1.3, using a strong cipher suite (e.g., TLS_AES_128_GCM_SHA256) and HSTS, for confidentiality and integrity of patient consultations.

- Secure file transfer: SFTP/FTPS replaces plain FTP for DICOM imaging transfer between buildings, addressing the stated unencrypted-file-transfer issue, with checksum verification for integrity.

- SMTP: critical alerts (patient monitor thresholds, device faults) are delivered via an SMTP relay with SMTP AUTH + STARTTLS and a secondary backup relay, addressing the stated email alert failures.

- SNMPv3: used instead of legacy SNMP versions for authenticated, encrypted monitoring of network devices and, where supported, medical IoT gateways, feeding into the same alerting path as SMTP for early fault detection.

### Systems Integration

The three subsystems interface as follows: Student A's VLAN and addressing plan defines the traffic classes that Student C's QoS policy schedules, and Student D's application services are placed on and secured within those same VLANs (e.g., the DNS and imaging file servers sit on their department VLANs and are reachable only through the addressing/routing plan Student A defined). A change in the addressing plan therefore requires corresponding updates to both the QoS policy's VLAN references and the application servers' network placement.

## 3.6 Trade-off Analysis

| Design Decision | Alternative Considered | Benefit | Disadvantage | Final Decision |
| --- | --- | --- | --- | --- |
| Hybrid partial-mesh backbone | Pure star / full mesh | Redundancy where it matters most (ICU link) without full-mesh cost | Higher cost than pure star | Hybrid selected |
| QUIC for real-time telemedicine | Plain UDP or TCP | Faster handshake, better loss recovery on wireless links | Less mature tooling/support than TCP | QUIC selected for video/telemetry |
| SNMPv3 over SNMPv2c | SNMPv2c (community strings) | Authenticated, encrypted management traffic | More complex initial configuration | SNMPv3 selected |

## 3.7 Sustainability, Ethics, Safety, Risk & Security

| Domain | Consideration | Evidence Evaluated | Impact on Final Decision |
| --- | --- | --- | --- |
| Sustainability | Energy use of always-on wireless sensors | Wi-Fi 6 Target Wake Time reduces radio-on duty cycle for battery-powered wearables | TWT enabled specifically for wearable/IoT SSIDs |
| Ethics | Patient data confidentiality | Plaintext services (FTP, SNMPv2c) expose patient-linked data in transit | Encrypted alternatives (SFTP, SNMPv3, TLS 1.3) mandated for all patient-data-adjacent services |
| Health & Safety | Delayed critical-care traffic risks patient harm | Retransmission storms and best-effort routing shown to delay ICU/emergency traffic under load | Priority queuing and redundant ICU-block links adopted |
| Security | Network segmentation and access boundaries | Flat network allows guest/patient Wi-Fi to reach clinical VLANs | Guest Wi-Fi isolated on its own VLAN, no route to clinical VLANs |

### Risk Register

| Risk | Likelihood | Severity | Risk Level | Mitigation | Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Single fibre link failure isolates a building | Medium | High | High | Redundant partial-mesh backbone links | Low |
| Congestion delays critical-care traffic at peak load | High | High | High | Priority queuing / QoS for VLAN 10/20 | Medium |
| Unauthorized access to patient data in transit | Medium | High | High | TLS 1.3, SFTP, SNMPv3 across all relevant services | Low |
| Wireless interference/packet loss near MRI | High | Medium | Medium | AP placement outside shielding; Wi-Fi 6 OFDMA | Low |

# CHAPTER 4: IMPLEMENTATION, TESTING & RESULTS, PROJECT MANAGEMENT

## 4.1 Development Approach & Resources

The design was implemented iteratively in Cisco Packet Tracer: the topology and addressing plan were built first (Student A), followed by routing and VLAN verification, then transport-layer QoS policy configuration and traffic-generation testing (Student C), and finally application-service configuration and security hardening (Student D). Wireshark was used throughout for packet-level verification, and all artefacts were committed to the team GitHub repository as they were completed.

| Resource | Purpose |
| --- | --- |
| Cisco Packet Tracer | Primary topology build, device configuration, Simulation Mode testing |
| Wireshark | Packet-level capture and verification of DNS/TLS/SMTP/SNMP/TCP behaviour |
| GitHub | Version control for configs, captures, diagrams, and the report |

## 4.2 Implementation & Modern Tools Used

### Topology and Device Configuration (Student A)

Core routers, distribution switches, and access switches were configured with the addressing and VLAN plan from Chapter 3; Wi-Fi 6 APs were configured with SSID-to-VLAN mapping (e.g., 'Hospital-Staff' to VLAN 40, 'Hospital-Guest' to VLAN 60).

### Transport & QoS Configuration (Student C)

Priority queuing was configured on core/distribution switches for VLAN 10/20 traffic; simulated TCP, UDP, and QUIC-like traffic flows were generated to compare behaviour under normal and congested conditions.

### Application Services & Security (Student D)

DNS, HTTPS/TLS 1.3, SFTP, SMTP, and SNMPv3 services were configured on their respective servers, each verified independently via Wireshark before integration testing.

| Tool / Software | Purpose | Stage Used |
| --- | --- | --- |
| Packet Tracer | Topology build, device config, Simulation Mode | Design & implementation |
| Wireshark | Protocol verification and performance capture | Testing & analysis |
| GitHub | Version control and evidence storage | Throughout |

## 4.3 Test Plan & Verification

| Requirement | Test Method | Acceptance Criterion |
| --- | --- | --- |
| Wireless packet loss (MRI zone) | Simulated ping/traffic test from AP near MRI room | < 0.5% packet loss |
| DNS resolution time | Wireshark capture of DNS query/response, filter dns | < 30 ms locally |
| Routing convergence | Simulated single-link failure in Packet Tracer, timed reconvergence | < 3 s |
| Priority traffic delay under load | Compare VLAN 10 vs VLAN 40 delay during simulated congestion | Priority traffic measurably lower delay |
| Application security | Wireshark capture of TLS/SFTP/SNMPv3 sessions | No plaintext credentials/content visible |

Filling in the Speciation Table

| Specification | Required Value | Achieved Value | Status |
| --- | --- | --- | --- |
| Routing convergence | <3 s | Enter measured failover time from Packet Tracer (e.g., 1.8 s) | Pass (or Fail) |
| Priority traffic delay under load | Priority traffic measurably lower delay | Enter measured average delays (e.g., VLAN 10: 12 ms vs VLAN 40: 85 ms) | Pass (or Fail) |
| Application security | No plaintext credentials/content visible | Enter verification summary (e.g., Encrypted via TLS/SFTP/SNMPv3) | Pass (or Fail) |
| DNS resolution time per building | (e.g., < 30 ms) | Enter measured DNS lookup response time | Pass / Fail |

## 4.4 Results

## Insert raw empirical data, visual graphs, and screenshots captured during your simulation runs:

## Routing Convergence: Time-stamped ping loss/recovery logs or timed reconvergence measurements following a simulated link failure in Packet Tracer.

## QoS / Traffic Delay: Comparative line graphs or bar charts showing latency/delay of priority traffic (VLAN 10) versus standard traffic (VLAN 40) during simulated network congestion.

## Application Security: Wireshark packet capture screenshots proving encrypted payloads (TLS, SFTP, SNMPv3) without readable plaintext credentials.

## DNS Resolution & Wireless Performance: DNS query resolution times per building and packet loss measurements in MRI-adjacent wireless zones before and after AP repositioning.

## 4.5 Data Analysis & Engineering Judgment

## Provide analytical context and engineering evaluation based on your results:

## Performance Analysis: Evaluate whether target thresholds were met (e.g., explaining why DNS resolution achieved or missed the 30 ms target, or discussing the magnitude of the delay gap created by priority queuing between VLAN 10 and VLAN 40).

## Root Cause Evaluation: Explain the physical or architectural reasons behind any performance discrepancies or deviations from expected values.

## Engineering Justification: Discuss trade-offs, design limitations of software simulations (e.g., Packet Tracer abstraction limits versus real-world hardware behavior), and how your design choices successfully met or compromised the hospital network's operational goals.

## 4.6 Comparison with Existing Solutions & Achievement of Objectives

| Objective | Evidence | Achievement |
| --- | --- | --- |
| Hierarchical, redundant topology design | Topology diagram, addressing/VLAN tables | [Fully/Partially] |
| Priority handling of critical-care traffic | QoS config, Wireshark delay comparison | [Fully/Partially] |
| Secured application services | TLS/SFTP/SNMPv3 configs and captures | [Fully/Partially] |

## 4.7 Strengths & Limitations

### Strengths

- Design decisions are each traced directly to a stated hospital requirement rather than made generically.

- Layered coverage (topology, transport QoS, application security) reflects how a real hospital network problem must be solved end-to-end.

- Wi-Fi 6 features (OFDMA, TWT) are applied purposefully to the specific problems named in the brief (dense clients, MRI interference, battery-powered sensors).

### Limitations

- All testing is simulation-based (Packet Tracer/Wireshark on emulated traffic); real RF interference near an MRI room and true 2,000-client wireless load cannot be fully replicated.

- QUIC support in Packet Tracer is limited, so QUIC behaviour is discussed analytically rather than fully captured in simulation.

- The design does not include a full HIPAA compliance audit — privacy considerations are addressed at a design-principle level only.

## 4.8 Project Planning, Roles & Budget

| Student | Assigned Responsibility | Actual Contribution |
| --- | --- | --- |
| HARSHENI.J | Overall network design, Packet Tracer topology, addressing plan, VLANs, wireless APs | Topology, addressing plan, VLAN table, AP design (Ch. 3–4) |
| JACK YANNIS.F | TCP/UDP/QUIC discussion, congestion & QoS analysis, Wireshark captures, performance metrics | QoS design, transport analysis, testing (Ch. 3–4) |
| JAICHITRA.G | DNS, HTTP/HTTPS+TLS, SMTP/FTP, SNMP; GitHub repo setup, report compilation, reflection | Application-layer design, GitHub setup, report compilation (Ch. 3–5) |

| Item | Estimated Cost | Actual Cost |
| --- | --- | --- |
| Simulation/software tools (Packet Tracer, Wireshark — free) | ₹0 | ₹0 |
| [Any additional resources used] |  |  |

# CHAPTER 5: CONCLUSION, FUTURE WORK AND REFLECTION

## 5.1 Conclusion

The project designed and simulated a hierarchical, secured hospital network across three buildings, addressing the stated wireless reliability, routing convergence, transport congestion, and application security problems through a hybrid topology, VLAN segmentation, Wi-Fi 6 wireless access, transport-layer QoS, and secured application services, validated using Packet Tracer and Wireshark evidence against the assignment's stated performance targets.

## 5.2 Future Work

- Extend testing to a physical or higher-fidelity emulation (e.g., GNS3/Mininet) to better model true RF interference and QUIC behaviour.

- Incorporate SDN/OpenFlow-based dynamic prioritisation rather than static QoS configuration, for adaptive response to changing traffic conditions.

- Conduct a formal HIPAA-alignment review with a healthcare compliance framework rather than design-principle-level privacy considerations.

## 5.3 Professional Learning & Individual Reflection

### New Knowledge Acquired

- Applying Wi-Fi 6 features (OFDMA, TWT) to a real deployment scenario rather than as abstract standard features.

- Designing and justifying a priority-queuing/QoS policy against measured or modelled delay, not just configuring it by default.

- Configuring and verifying authenticated/encrypted application services (TLS 1.3, SFTP, SNMPv3) and reading their handshake/session behaviour in Wireshark.

### Engineering & Professional Skills Developed

- Trade-off evaluation using a weighted decision matrix rather than informal preference.

- Evidence-based engineering documentation — tying every design claim to a specific, labelled test or capture.

- Cross-layer systems thinking — recognising how a change in addressing/VLANs propagates to QoS policy and application server placement.

### Individual Reflection

### My Technical Role: Highlight your specific contribution—such as configuring encrypted management protocols (SNMPv3, SFTP), conducting packet captures in Wireshark, and optimizing wireless access point placement in high-interference areas.

### Challenges & Solutions: Explain how you tackled difficulties with protocol configuration or wireless dead-zones, detailing the troubleshooting steps you took to achieve pass criteria.

### Collaboration: Discuss how your security configurations and wireless placement integrated with Student A and Student C's routing and QoS delay configurations.

### Key Takeaway: Summarize what you learned about secure network design and wireless optimization in critical healthcare environments.

REFERENCES

[1] IEEE 802.11ax-2021, "IEEE Standard for Information Technology — Telecommunications and Information Exchange between Systems — Local and Metropolitan Area Networks — Specific Requirements Part 11: Wireless LAN MAC and PHY Specifications Amendment: Enhancements for High-Efficiency WLAN."

[2] RFC 8446, "The Transport Layer Security (TLS) Protocol Version 1.3," IETF.

[3] RFC 3411–3418, "An Architecture for Describing SNMP Management Frameworks," IETF (SNMPv3).

[4] RFC 1035, "Domain Names — Implementation and Specification," IETF (DNS).

[5] RFC 5321, "Simple Mail Transfer Protocol," IETF (SMTP).

[6] Cisco Systems, "Cisco Packet Tracer Documentation."

[7] Wireshark Foundation, "Wireshark User's Guide."

[8] United Nations, "Sustainable Development Goals 3, 9, and 11."

[9] AI-assisted drafting tool (Claude, Anthropic) was used to help structure and draft sections of this report; all technical design decisions and content were reviewed by the student authors.

APPENDICES

Appendix A – Detailed Calculations (subnetting, queuing model) — insert here.

Appendix B – Engineering Drawings / Schematics (topology diagrams) — insert here.

Appendix C – Source Code / Configuration Repository Information — GitHub repository link here.

Appendix D – Datasheets — insert if applicable.

Appendix E – Experimental / Raw Data (Wireshark captures, simulation logs) — insert here.

Appendix F – Ethics / Institutional Approval — where applicable.

Appendix G – Project Meeting / Progress Records — insert here.

Appendix H – User/Industry Feedback — where applicable.

Appendix I – Publications / Patents / Competitions / Product Outcomes — where applicable.

Appendix J – Individual Contribution Evidence — commit history / screenshots per member.